Report No.: 105/EMC/TA/2021

ELECTROMAGNETIC COMPATIBILITY (EMC) TEST REPORT

| Applicant’s Data | : | PT. GERBANG TEKNOLOGI INFORMASI SOHO Podomoro City (NEO SOHO) 40th Floor Unit 09, Jalan Letjen S Parman Kav 28, JAKARTA BARAT, 11470, Indonesia |
| --- | --- | --- |
| Equipment Data |  |  |
| Equipment name | : | GAS METER |
| Brand | : | CHINT |
| Type/Model | : | ZG 1.6 IOT GAS METER |
| Capacity | : | LoRa 920 – 923 MHz |
| Serial number | : | 190000010072 |
| Made in | : | SINGAPORE |

General Statement

- This test report is only valid for the Equipment Data which is stated above.

- Test reference: 	IEC CISPR 32:2015 (Electromagnetic Compatibility of Multimedia

Equipment – Emission Requirements)

Testing Information

| Start Date | : | 10 December 2021 |  |  |
| --- | --- | --- | --- | --- |
| End Date | : | 15 December 2021 |  |  |
| Tested By | : | Infrastructure Quality Assurance Laboratory - Test Lab |  |  |
| Agreed by: |  |  |  | Reviewed by: |
| I GEDE ASTAWA Senior Manager Infrastructure Assurance |  |  |  | ADI PERMADI Manager Lab Infrastructure QA |

TABLE OF CONTENTS

TEST RESULT SUMMARY	3

GENERAL TEST INFORMATION	4

FEATURE OF EUT	4

EUT COMPLIMENTARY ITEMS/EQUIPMENT/DEVICES	4

LIST OF MEASURING EQUIPMENT	5

MEASUREMENT UNCERTAINTIES	6

ENVIRONMENTAL CONDITION	6

ADDITIONAL STATEMENT	6

EMISIONS TEST RESULTS	8

RADIATED EMISSION	8

# TEST RESULT SUMMARY

- Emission Test Result

| Emission Tests |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Standard | Item Test | Detector | Measurement Results (dBuV) | Suitability |
| 1. | CISPR 32:2015, Class B Radiated Emissions | 30 – 230 MHz | Quasi-Peak | 38,50* | Pass |
|  |  | 230 – 1000 MHz |  | - | Pass |
|  |  | 1000 – 3000 MHz | Peak | - | Pass |
|  |  | 3000 – 6000 MHz |  | 46,40 | Pass |
|  |  | 1000 – 3000 MHz | Average | - | Pass |
|  |  | 3000 – 6000 MHz |  | 46,40 | Pass |
| Note: *) Using peak detector according to the procedure at point 3 (Peak value less than Quasi Limit) |  |  |  |  |  |

# GENERAL TEST INFORMATION

- Equipment Photo

### FEATURE OF EUT

- Feature of EUT

| No. | Feature | Value |
| --- | --- | --- |
| 1. | LoRa | 920 - 923 MHz |

### EUT COMPLIMENTARY ITEMS/EQUIPMENT/DEVICES

- EUT Complimentary Items/Equipment/Devices

| No. | Item |
| --- | --- |
| 1. | - |

### LIST OF MEASURING EQUIPMENT

- List of Measuring Equipment

| No. | Name | Brand | Type/Model | Serial Number | Asset Number |
| --- | --- | --- | --- | --- | --- |
| 1. | SIGNAL & SPECTRUM ANALYZER | ROHDE & SCHWARZ | FSW43 | 103952 | 134/TRA |
| 2. | EMC SOFTWARE MEASUREMENT | ROHDE & SCHWARZ | VERSION 10.50.40 | PRODUCT ID: 1040-0 | 156/TRA |
| 3. | SEMI ANECHOIC CHAMBER | MEASUREMENT ENGINEERING | 3 METER STANDARD CHAMBER | - | 148/TRA |
| 4. | CONTROLLER EM 1000 | EM ELECTRONICS | EM1000 | 060873 | 152/TRA |
| 5. | ANTENNA EMC (30 MHz - 6 GHz) | AARONIA | - | 201746 | 146/TRA |
| 6. | DRG HORN ANTENNA | AH SYSTEM, INC | SAS-571 | 756 | 150/TRA |

### MEASUREMENT UNCERTAINTIES

- Measurement Uncertainties

| Measurement Uncertainties |  |  |
| --- | --- | --- |
| Test Item | Frequency | Uncertainty |
| Radiated Emission | 30-1000 MHz | 1.05 dB |
|  | 1000-6000 MHz | 1.05 dB |
| Note: ISO/IEC 17025:2015 requires that an estimate of the measurement uncertainties associated with emissions and immunities test results are included in the report. The measurement uncertainties given below are based on a 95% confidence level (based on coverage factor K=2). |  |  |

### ENVIRONMENTAL CONDITION

- Environmental Condition

| Metrics | Value |
| --- | --- |
| Temperature | 22 °C |
| Relative Humidity | 60 % |
| Voltage | 219 V |

### ADDITIONAL STATEMENT

-

EUT TESTING DIAGRAM

LoRa Gas Meter

- EUT Testing Diagram

# EMISIONS TEST RESULTS

### RADIATED EMISSION

- LIMIT

- Requirement Limit for Class B Radiated Emission up to 1 GHz

| No | Frequency Range MHz | Measurement |  |  | Class B limits dB(V/m) |
| --- | --- | --- | --- | --- | --- |
|  |  | Facility | Distance M | Detector Type / Bandwidth |  |
| 1. | 30 to 230 | OATS/SAC | 3 | Quasi Peak / 120 kHz | 40 |
| 2. | 230 to 1000 | OATS/SAC | 3 | Quasi Peak / 120 kHz | 47 |

- Requirement Limit for Class B Radiated Emission Above 1 GHz

| No | Frequency Range MHz | Measurement |  |  | Class B limits dB(V/m) |
| --- | --- | --- | --- | --- | --- |
|  |  | Facility | Distance M | Detector Type / Bandwidth |  |
| 1. | 1000 to 3000 | FSOATS | 3 | Average / 1 MHz | 50 |
| 2. | 3000 to 6000 |  |  | Average / 1 MHz | 54 |
| 3. | 1000 to 3000 | FSOATS | 3 | Peak / 1 MHz | 70 |
| 4. | 3000 to 6000 |  |  | Peak / 1 MHz | 74 |

- CONFIGURATION TEST

- Configuration Test

- PROCEDURE

- Test Procedure for Radiated Emission

- RESULTS

  - Test Result Up to 1 GHz

- Radiated Emission Up to 1 GHz Resume - Resume Result Radiated Emission Up to 1 GHz Detector; Frequency (MHz); Peak Level (dBuV); Class B QP Limit (dBuV); Margin (dB); Polarization; Remark. Peak; 30.431111; 34,30; 40; 5,70; H; -. Peak; 130.287222; 38,50; 40; 1,50; V; -. Peak; 225.670556; 35,70; 40; 4,30; H; -. Peak; 921.106667; 92,70; 47; -45,70; V; Work Operation.

  - Test Resul Above 1 GHz

- Radiated Emission Above 1 GHz Resume - Resume Result Radiated Emission Above 1 GHz Detector Peak Detector; Frequency (MHz); Peak Level (dBuV); Class B PK Limit (dBuV); Margin (dB); Polarization; Remark. Peak; 4.926.000.000; 46,40; 74; 27,60; H; -. Resume - Resume Result Radiated Emission Above 1 GHz Detector Average Detector; Frequency (MHz); AV Level (dBuV); Class B AV Limit (dBuV); Margin (dB); Polarization; Remark. Average; 4.926.000.000; 46,40; 54; 7,60; H; -.

- FIGURE TEST

Antenna Above 1 GHz Antenna Below 1 GHz

- Figure Test of Radiated Emission

- Figure of Device Under Test

- Figure of Device Under Test